# JLG ES2632 SCISSOR LIFT   OPERATING INSTRUCTIONS & SPECIFICATIONS

Quick Start Guide

The ground control station and platform control station emergency stop buttons (RED) must be set to “ON” in order to operate this machine.

- Travelling and steering – Place the key selector switch on the ground control station to platform operation. On the platform control station, position the lift/drive switch to the drive position

  - To move unit forward and backward: Grab the joystick and pull the trigger on the back side. Push forward to move forward and pull backward to drive the unit backward.

  - To steer the unit: At the top of the joystick there is a rocker switch. Push the left side of the switch to turn the wheels left and push the right side of the switch to turn the wheels to the right.

- Raising and lowering platform – On the platform control station, position the lift/drive switch to the lift position

  - To raise and lower platform – Grab the joystick and squeeze the trigger on the back side. Pulling the joystick will raise the platform and pushing the joystick will lower the platform.

- When unit is not in use – Make sure to push the emergency stop button when the machine is not in use. This will preserve the battery and avoid any accidents.

- Getting power to the platform – If you are using electric tools while in the air, there is a power outlet on the platform. In order to power the outlet, you must grab the plug from the front of the machine and plug it into a power source. This will provide the power to the platform outlet.

- Charging the Lift – At the base of the rear of the unit (where you climb onto the platform) there is a charging cord. Plug it into a power source to charge the unit.

- Indoor/Outdoor – On the platform control box, to the right of the horn button, there is another button with a ‘T’ on it. When using the lift indoors, the button should be pressed, displaying a green ‘T’ next to the power bars. This will limit the lift to about 75% of the maximum height. When using outdoors, press the button again and it will show a yellow ‘T’ allowing you to use the maximum lift height.

# JLG ES2632 SCISSOR LIFT

OPERATING INSTRUCTIONS & SPECIFICATIONS

Unit Specifications

| Ground Clearance | 4” |
| --- | --- |
| Machine Height | 7’ 4” |
| Machine Height (Rails Folded) | 5’ 10” |
| Machine Length | 7’ 10” |
| Machine Width | 2’ 8” |
| Machine Weight | 4,400 lbs |
| Platform Extension Length | 2’ 10” |
| Platform Height – Lowered | 3’ 8” |
| Platform Railing Height | 3’ 8” |
| Platform Size | 25.4 in x 84 in |
| Wheel Base | 6’ 2” |
| Platform Height | 25’ 6” |
| Working Height | 31’ 6” |
| Gradeability | 25% |
| Platform Capacity | 507 lbs |
| Platform Capacity – Extension | 265 lbs |